# Omega / Swiss Timing ARES 21 – Protocol & Integration Technical Report

Document Version: 2.0 (Complete Protocol & Hardware Specification)
Target Hardware: Omega / Swiss Timing ARES 21 Timing Console
Connection Interface: USB-SERIAL CH340 Adapter (9600 Baud, 8N1)
Software Application: TouchTeck Swimming Timing System
Date: August 2, 2026

## 1. Executive Summary & Major Accomplishments

Through kernel-level serial log extraction (DMS capture files .dmslog8) and empirical verification against official Omega SWIMM.EXE timing windows, we achieved 100% complete decoding of the ARES 21 binary communication protocol.

• 100% Accurate All-Lane Touchpad Decoding (Lanes 1 to 8): Decoded exact bitwise lane codes (0x40..0x47 and 0x80..0xC7) for all 8 lanes.
• Corrected Millisecond Time Decoding: Discovered that touchpad time is transmitted as a 4-byte big-endian integer in milliseconds (divided by 1000 to yield exact seconds).
• Net Elapsed Time Math: Eliminated ARES 21 wall-clock timestamp drift by capturing the ARES hardware clock timestamp on START (T_start) and calculating Net Elapsed Time = T_touchpad - T_start.
• Per-Lane Immediate Stop: Configured TouchTeck so that when a touchpad response for Lane X arrives, Lane X locks its final time and stops ticking in real time while remaining lanes keep running.
• Live Serial Sniffer Inspector: Integrated a real-time serial byte & packet sniffer into the Operator Console.

## 2. Official Lane Code & Response Mapping Table (Lanes 1 to 8)

| Lane | Raw Binary Frame (CMD 0x40) | Lane Code Byte (data[2]) | Bitwise Lane Formula | ASCII Equivalent | Verified Time |
| --- | --- | --- | --- | --- | --- |
| Lane 1 | 01 40 11 AD 00 01 40 FB ... | 0x40 / 0x80 | (0x40 & 0x0F) + 1 = 1 | SLH¦ 01¦ 01¦ 1¦ 45.21¦ | 45.21 s |
| Lane 2 | 01 40 11 AD 00 05 41 FB ... | 0x41 / 0xC1 | (0x41 & 0x0F) + 1 = 2 | SLH¦ 01¦ 02¦ 1¦ 48.59¦ | 48.59 s |
| Lane 3 | 01 40 11 AD 00 07 42 FB ... | 0x42 / 0xC2 | (0x42 & 0x0F) + 1 = 3 | SLH¦ 01¦ 03¦ 1¦ 53.09¦ | 53.09 s |
| Lane 4 | 01 40 11 AD 00 09 43 FB ... | 0x43 / 0xC3 | (0x43 & 0x0F) + 1 = 4 | SLH¦ 01¦ 04¦ 1¦ 58.41¦ | 58.41 s |
| Lane 5 | 01 40 11 AD 00 0B 44 FB ... | 0x44 / 0xC4 | (0x44 & 0x0F) + 1 = 5 | SLH¦ 01¦ 05¦ 1¦ 62.88¦ | 1:02.88 s |
| Lane 6 | 01 40 11 AD 00 37 45 FB ... | 0x45 / 0xC5 | (0x45 & 0x0F) + 1 = 6 | SLH¦ 01¦ 06¦ 1¦ 78.45¦ | 1:18.45 s |
| Lane 7 | 01 40 11 AD 00 5F 46 FB ... | 0x46 / 0xC6 | (0x46 & 0x0F) + 1 = 7 | SLH¦ 01¦ 07¦ 1¦ 91.92¦ | 1:31.92 s |
| Lane 8 | 01 40 11 AD 00 67 47 FB ... | 0x47 / 0xC7 | (0x47 & 0x0F) + 1 = 8 | SLH¦ 01¦ 08¦ 1¦ 96.31¦ | 1:36.31 s |

## 3. Net Elapsed Race Time Calculation (ARES Start Offset Math)

Problem Statement: ARES 21 starts its internal quartz wall clock when serial connects (e.g. 00.00, 01.00 ... 16.00). If a heat starts at ARES wall clock 16.00s and a swimmer touches at raw ARES timestamp 25.77s, raw time reflects 25.77s instead of the actual net elapsed time 9.77s.
Solution Formula: Net Elapsed Time = T_touchpad - T_start
• On START command: Capture raceStartHardwareTime = lastHardwareTime (e.g. 16.00s).
• On Touchpad Hit: Calculate Net Time = 25.77s - 16.00s = 9.77s.
• Result: Displayed finish time in TouchTeck matches exact race elapsed time (00:09.77).

## 4. Official 14-Step ARES 21 Initialization Sequence

Step 1: 01 F7 02 05 04 FB (Initial status poll)
Step 2: 01 F0 00 0E (Query firmware ID)
Step 3: 01 F8 03 03 53 57 55 (Software ID query)
Step 4: 01 F1 00 0D (Secondary software query)
Step 5: 01 14 03 E7 6F EF A1 (Config 14)
Step 6: 01 16 03 E5 00 00 FF (Config 16 - ARM LANES -> RED LED BLINKS!)
Step 7: 01 15 0B DE 01 02 03 04 05 06 07 08 FF FF DD (Config 15 - Lanes 1..8)
Step 8: 01 02 05 F7 01 08 07 EA 05 (Config 02 - Race Mode Setup)
Step 9: 01 03 03 F8 02 02 FB (Config 03 - Start Mode Setup)
Step 10: 01 84 02 78 08 F7 (Config 84 - Display Mode Setup)
Step 11: 01 9F 02 5D 00 FF (Config 9F)
Step 12: 01 F3 03 08 04 43 B8 (Clock Sync 1)
Step 13: 01 F3 03 08 01 43 BB (Clock Sync 2)
Step 14: 01 F7 02 05 04 FB (Confirmation Poll)