TOUCHTECK  for OMEGA

OMEGA / SWISS TIMING ARES 21

Comprehensive Master Protocol & Integration Guide

Serial Interface, Binary Framing, T1/T2 Arbitration & Starter Protocol — Fully Consolidated Edition

| Document Version | 4.0 — Fully Consolidated Master Reference (All Sources Merged, Verification-Flagged) |
| --- | --- |
| Project | TouchTeck Swimming Timing Software |
| Target Hardware | Omega / Swiss Timing ARES 21 Timing Console & Simulator |
| Connection Interface | USB-SERIAL CH340 / RS-232 DB9 Serial (9600 Baud, 8N1) |
| Software Application | TouchTeck Swimming Timing System |
| Sources Merged | ARES21_MAIN_DATA_SOFTWARE, Master Protocol Doc v3.0, Protocol Documentation v2.1, Starter Master Documentation, Starter Test Data Analysis, T1/T2 Arbitration Review |
| Date | August 5, 2026 |

HOW TO READ THIS DOCUMENT This document merges six separate source documents (the original ARES21_MAIN_DATA_SOFTWARE plus five newer drafts covering the starter, T1/T2 arbitration, and master protocol) into one reference. Every finding is labeled VERIFIED, PARTIALLY VERIFIED, or UNVERIFIED based on whether it was directly confirmed through joint kernel-level capture analysis cross-checked against ARES v2.06's on-screen output. Where source documents conflicted with each other, both claims are shown side-by-side rather than silently resolved. Treat UNVERIFIED and CONTRADICTION items as design intent or draft notes, not confirmed hardware behavior.

# 1. Executive Summary

## Verified

- Physical serial parameters: 9600 baud, 8N1, RTS/DTR cleared.

- 14-step connection handshake byte sequence — present in every capture.

- T1 touchpad lane formula: lane = (payload byte & 0x0F) + 1, for byte range 0x40–0x47. Confirmed across 3+ independent sessions, including reversed finish-order edge cases.

- Millisecond timestamp decoding: 4-byte big-endian ÷ 1000. Exact match against ARES v2.06 across 12+ touch events.

- A single physical tap can generate 2–3 near-duplicate CMD 0x40 frames within ~50ms — parser-level dedup required.

- CMD 0x41 exists as a structurally distinct frame type from CMD 0x40, observed twice during a starter-gun test — consistent with being a dedicated starter-trigger channel, but only 2 samples exist.

## Corrected

- No single command "arms" the device or triggers the blinking LED. The ARES 21 was observed blinking immediately upon connection, before any specific init step, in a controlled test. Step 6 (Config 16) is sent every session as part of standard init, but a direct causal link to the LED has been disproven, not confirmed. See Section 5.

## Unverified / Not Yet Tested

- T2 backup-button hardware byte range — see CONTRADICTION in Section 9: two source documents disagree (0x20–0x27 vs. 0x81–0x88/0xC1–0xC8).

- Checksum formula (HCHECK/DCHECK) — proposed but never tested against real captured checksum bytes.

- Secondary lane byte range 0x80–0xC7 — never observed in any joint capture; all confirmed lane bytes fell within 0x40–0x47 only.

- 0x32 "clock drop = gun trigger" detection method, and 0x25/0x26 disarm commands — not independently tested.

- Starter Test Analysis's per-run lane list — 4 of 6 listed runs show identical results ("Lanes 3, 5, 8"), which looks like a data/copy artifact rather than genuinely distinct measurements. See Section 10.

# 2. System Architecture — Node.js WebSocket Bridge (Implementation Notes)

Described in ARES21_STARTER_MASTER_DOCUMENTATION.md as the solution to the browser-refresh disconnect problem discussed earlier in this project. This is software architecture, not a hardware protocol claim, and is internally consistent with the recommended fix. This section documents how the bridge is intended to behave end-to-end, since the actual serial-bridge.js source was not provided for direct verification — treat the code samples below as a reference implementation pattern, not a byte-for-byte transcript of the real file.

Physical ARES 21 Console (CH340 USB-Serial, VID 1A86 / PID 7523) ¦ RS-232 / USB (9600 Baud 8N1) v Node.js Serial Bridge (serial-bridge.js) Owns the SerialPort object exclusively Runs the 14-step init sequence once, on process start WebSocket Server on ws://localhost:8787 ¦ WebSocket JSON/binary stream (<10ms) v TouchTeck React Web App (App.tsx / serialDriver.ts) One WebSocket client per open browser tab

## 2.1 Why the bridge exists

The Web Serial API ties a SerialPort object to the lifetime of the page that opened it. A refresh, navigation, or crash tears the connection down completely, and Chrome does not allow two independent contexts to hold the same port at once. The bridge solves this by moving port ownership out of the browser entirely: a small, long-running Node.js process opens COM1 once and keeps it open indefinitely, independent of any tab's lifecycle.

## 2.2 What the bridge process does on startup

- Opens the configured COM port (e.g., COM1) at 9600 baud, 8N1, using the node serialport package.

- Runs the full 14-step initialization handshake (Section 5) exactly once, immediately after the port opens.

- Starts a lightweight WebSocket server (e.g., using the ws package) listening on a fixed local port such as 8787.

- Begins continuously reading raw bytes from the serial port, buffering and splitting them into individual frames using the SOH (0x01) / LEN framing described in Section 4.

## 2.3 How data is returned to the browser — message format

Each decoded event (touchpad hit, ASCII status line, starter signal, keepalive) is converted into a small JSON object and broadcast over the WebSocket to every currently connected client. A representative message shape:

{ "type": "TOUCH", "method": "T1", "lane": 4, "timeMs": 61412, "raw": "01400 11ad000...", // original hex, optional, for debugging "seq": 27, // bridge-assigned monotonic counter "ts": 1785900000000 // bridge wall-clock time (Date.now()) }

Other message types follow the same envelope, varying the type field: "START" (race start), "KEEPALIVE" (periodic heartbeat), "STATUS" (raw SLH/TLH lines), and "CONN_STATE" (bridge-to-client connection status, e.g. when the physical serial port itself disconnects or errors). Keeping a single flat JSON envelope for every message type makes the client-side handler a single switch statement rather than several different parsers.

## 2.4 Client-side connection (replacing direct Web Serial calls)

Instead of serialDriver.ts calling navigator.serial directly, it opens a plain WebSocket to the bridge and listens for messages:

const ws = new WebSocket('ws://localhost:8787'); ws.onmessage = (event) => { const msg = JSON.parse(event.data); switch (msg.type) { case 'TOUCH': this.emitTiming(msg.method, msg.lane, msg.timeMs / 1000); break; case 'START': this.emitStart(msg.ts); break; case 'KEEPALIVE': this.lastHeartbeat = msg.ts; break; case 'CONN_STATE': this.handleBridgeConnState(msg); break; } }; ws.onclose = () => { // Only the thin WebSocket dropped — the hardware connection in the // bridge process is unaffected. Retry with backoff. setTimeout(() => this.connectToBridge(), 500); };

## 2.5 Reconnection behavior across a page refresh

- On refresh, the old WebSocket closes automatically as the page unloads — this does not touch the serial port at all, since the bridge process (a separate OS process) owns it.

- The new page load opens a fresh WebSocket to the same ws://localhost:8787 endpoint. Typical reconnect time is a few milliseconds to low tens of milliseconds — effectively instant compared to a full serial re-handshake.

- Because the bridge never closed the port, no re-initialization (Section 5's 14 steps) is needed on reconnect — the bridge can simply replay its current known state (e.g., "race currently running, started at ts=X") to the newly-connected client so the UI can catch up immediately.

- If the bridge process itself is not running (e.g., first-time setup, or it crashed), the client's WebSocket connection attempt fails immediately — this is the trigger for the Web Serial API fallback path.

## 2.6 Multi-tab / multi-client behavior

Because the WebSocket server can accept multiple simultaneous connections, several browser tabs — an Operator Desk, a public Scoreboard view, a Reports tab — can each hold their own WebSocket to the same bridge and receive identical, synchronized events in real time, without any of them competing for the actual serial port. This is not possible with direct Web Serial API access, where only one page can hold the port at a time.

## 2.7 Shutdown / "only disconnect on logout" behavior

Since the bridge is a separate long-running process, the serial connection persists across any number of page refreshes by design. To satisfy a "disconnect only on logout" requirement, the logout action in the web app should send an explicit shutdown message to the bridge (e.g., a small HTTP endpoint or a dedicated WebSocket message type such as {"type":"SHUTDOWN"}), which the bridge process listens for and uses to gracefully close the serial port and exit, or return to an idle/disarmed state, rather than relying on any browser-side event to do so.

## 2.8 Operational summary

- Page refresh reconnects only the WebSocket (milliseconds) — the Node process keeps COM1 open continuously.

- Multiple browser tabs can share the live feed simultaneously.

- Falls back to native Web Serial API if the bridge process is not running.

- This entire section describes intended/recommended behavior consistent with the project's stated goals — it has not been verified against the actual running serial-bridge.js source, since that file was not shared for direct review.

# 3. Serial Communication — Physical Parameters (Verified)

| Parameter | Setting | Note |
| --- | --- | --- |
| Baud Rate | 9600 | VERIFIED — device negotiates 19200 → 9600 on connect. |
| Data Bits | 8 | VERIFIED — standard 8N1 framing. |
| Parity | None | VERIFIED |
| Stop Bits | 1 | VERIFIED |
| Flow Control | None | VERIFIED — RTS/DTR explicitly cleared after connect. |

# 4. Frame Structure

General framing: SOH (0x01) | Command byte | Length byte | Payload | Checksum byte.

- CMD 0x81 — ASCII event wrapper: 01 11 39 38 02 <text> 04. Carries START, START_1, TI0/TI1/TI2, TLH, SLH status lines. VERIFIED, repeats ~every 6s.

- CMD 0x40 — Binary touchpad event: 01 40 11 <17-byte payload> <checksum>. VERIFIED structure.

- CMD 0x41 — Structurally identical to 0x40 but with distinct marker bytes; PARTIALLY VERIFIED, see Section 9.

NOTE ON PAYLOAD INDEXING (RESOLVED) Earlier drafts used two different conventions: one treats the byte after LEN as "HCHECK" with PAYLOAD[0] starting after it; another treats that same byte (0xAD/0xAC) as PAYLOAD[0] itself. Cross-checked directly: both conventions point to the identical real byte for the lane field (absolute frame position 6, e.g. 0x40 for lane 1). This is not a discrepancy — just two labeling schemes for the same data. Whichever convention your actual serialDriver.ts code uses, confirm data[0] in code corresponds to the byte immediately after LEN (0xAD/0xAC), which resolves both schemes consistently.

# 5. Connection Handshake Sequence (Verified Bytes)

| # | Step | Hex Bytes | Purpose |
| --- | --- | --- | --- |
| 1 | Status Poll | 01 F7 02 05 04 FB | Initial handshake / poll request |
| 2 | Query Firmware | 01 F0 00 0E | Requests firmware identity |
| 3 | Software ID ("SWU") | 01 F8 03 03 53 57 55 | Software handshake header |
| 4 | Secondary Query | 01 F1 00 0D | Secondary console query |
| 5 | Config 14 | 01 14 03 E7 6F EF A1 | Protocol subsystem configuration |
| 6 | Config 16 | 01 16 03 E5 00 00 FF | Sent as part of standard sequence — see correction note below |
| 7 | Config 15 (Lane Map) | 01 15 0B DE 01..08 FF FF DD | Configures pool lanes 1–8 |
| 8 | Config 02 (Race Setup) | 01 02 05 F7 01 08 07 EA 05 | Sets race mode parameters |
| 9 | Config 03 (Start Mode) | 01 03 03 F8 02 02 FB | Configures start trigger mode |
| 10 | Config 84 (Display) | 01 84 02 78 08 F7 | Prepares scoreboard display mode |
| 11 | Config 9F | 01 9F 02 5D 00 FF | Auxiliary configuration |
| 12 | Clock Sync 1 | 01 F3 03 08 04 43 B8 | Syncs hardware timer clock (part 1) |
| 13 | Clock Sync 2 | 01 F3 03 08 01 43 BB | Syncs hardware timer clock (part 2) |
| 14 | Keepalive Poll | 01 F7 02 05 04 FB | Repeats ~every 6s as ongoing keepalive |

CORRECTION — NO SINGLE COMMAND "ARMS" THE DEVICE Multiple source documents state that Step 6 (Config 16, 01 16 03 E5 00 00 FF) is the specific command that arms the console and triggers the red/green LED to blink. This was directly tested and disproven in a controlled capture: the ARES 21 was already blinking from the moment of connection, before Step 6 — and before any single step could be isolated as the trigger. The device enters and remains in an active state as long as it continues receiving valid, continuous, correctly-framed traffic at the correct baud rate. Step 6 is sent every session as part of standard initialization, but no isolated test has shown it alone causes the LED behavior.

# 6. Reverse-Engineered Command Code Reference

Consolidated from ARES21_STARTER_MASTER_DOCUMENTATION.md, with verification status added:

| Command | Name | Hex Sequence | Claimed Effect | Status |
| --- | --- | --- | --- | --- |
| 0x16 | Arm Lanes / Green Light | 01 16 03 E5 00 00 FF | Turns on ready light, arms lanes | PARTIALLY VERIFIED — sent every session; "arms/triggers blink" causal claim disproven, see Sec. 5 |
| 0x25 / 0x26 | Disarm Hardware | 01 25 02 ... / 01 26 02 D6 00 FF | Extinguishes ready light, disarms | UNVERIFIED — not captured/tested jointly |
| 0x31 | Mode Enable Handshake | 01 31 02 CB 01 FE | Prepares registers for heat config | VERIFIED byte sequence present in captures; functional claim not independently tested |
| 0x14 | Timer Configuration | 01 14 03 E7 6F EF A1 | Configures timer precision mode | VERIFIED byte sequence present in every capture |
| 0x15 | Lane Mapping | 01 15 0B DE 01..08 FF FF DD | Assigns T1/T2 sensors to lanes 1–8 | VERIFIED byte sequence present in every capture |
| 0x40 / 0x41 | Touchpad / Starter Trigger | see Sections 6–9 | Touch event / starter gun signal | VERIFIED (0x40, lanes); PARTIALLY VERIFIED (0x41, only 2 samples captured) |
| 0x32 | Running Clock Heartbeat | 01 32 05 C7 ... | Periodic hardware clock tick broadcast | VERIFIED as a periodic heartbeat; "clock-drop = gun trigger" detection method UNVERIFIED |
| 0x02 | Mode Setup | 01 02 05 F7 ... | Sets mode parameters | VERIFIED byte sequence present in captures |
| 0x03 | Start Mode Setup | 01 03 03 F8 02 02 FB | Configures start trigger mode | VERIFIED byte sequence present in captures |
| 0xF3 | Clock Synchronization | 01 F3 03 08 04/01 43 B8/BB | Resets/syncs internal clock counters | VERIFIED byte sequence present in captures |
| 0xF7 | Status Poll Keepalive | 01 F7 02 05 04 FB | Periodic keepalive poll | VERIFIED — observed repeating ~every 6s (not 3.5s as one source claimed) |

# 7. T1 Touchpad Lane Decoding (VERIFIED)

lane = (payload_byte & 0x0F) + 1     [byte range 0x40-0x47]

Confirmed across three independent capture sessions, cross-matched exactly against ARES v2.06's on-screen display, including cases where finish order did not match lane order.

| Lane | Lane Byte | Verified Time Example |
| --- | --- | --- |
| 1 | 0x40 | 45.21 s |
| 2 | 0x41 | 48.59 s |
| 3 | 0x42 | 53.09 s |
| 4 | 0x43 | 58.41 s |
| 5 | 0x44 | 1:02.88 s |
| 6 | 0x45 | 1:18.45 s |
| 7 | 0x46 | 1:31.92 s |
| 8 | 0x47 | 1:36.31 s |

SECONDARY RANGE 0x80–0xC7 — STILL UNVERIFIED Repeated across multiple source documents as an "alternate" lane byte range (0x80/0xC0-0xC7). This range was not observed in any joint capture session — every confirmed lane byte fell within 0x40-0x47 only. Do not rely on this range until independently reproduced against a known on-screen lane assignment.

DO NOT USE THE FINISH-ORDER BYTE AS LANE An earlier parsing approach used the finish-place counter (1st, 2nd, 3rd place finisher) as the lane number. This looks correct only when swimmers finish in lane order, and was shown to mislabel lanes when finish order differs from lane order (tested failure case: lanes 6, 7, 8 finishing in reverse placement order).

# 8. Time Value Decoding (VERIFIED)

Format: 4-byte big-endian integer, in milliseconds. Divide by 1000 for seconds. Byte offset is not always fixed — recommended method: scan for a 4-byte value repeated twice in a row, rather than a hardcoded offset.

| Lane | Decoded | Official ARES v2.06 Display |
| --- | --- | --- |
| 1 | 39.070 s | 39.07 |
| 2 | 51.131 s | 51.13 |
| 3 | 56.521 s | 56.52 |
| 4 | 61.412 s | 1:01.41 |
| 5 | 65.182 s | 1:05.18 |
| 6 | 118.810 s | 1:58.81 |
| 7 | 95.937 s | 1:35.93 |
| 8 | 75.361 s | 1:15.36 |

# 9. T1 vs T2 Sensor Byte Decoding — CONTRADICTION BETWEEN SOURCES

UNRESOLVED CONTRADICTION — DO NOT SHIP EITHER VALUE WITHOUT TESTING Two source documents disagree on the T2 backup-button byte range:

| Source Document | T2 Byte Range Claimed | Indexing |
| --- | --- | --- |
| ARES21_T1_T2_Arbitration_Review.md | 0x20 – 0x27 | 0-indexed, (b & 0x07) + 1 |
| ARES21_Master_Protocol_Doc.md (Sec. 9) | 0x81 – 0x88 / 0xC1 – 0xC8 | 1-indexed, (b & 0x0F), no +1 |

Neither range was observed in any joint capture session performed together — this entire T2 pathway remains unverified regardless of which range is correct. Recommended resolution: run an isolated capture pressing only the T2 backup button (no touchpad, no starter), the same way lane and starter tests were isolated, and read the actual byte range directly from that capture rather than trusting either document.

## Driver Implementation — as provided (ARES21_T1_T2_Arbitration_Review.md version, 0x20-0x27)

private parseCmd40(data: number[], forceMethod?: 'T1' ¦ 'T2') { if (data.length < 12) return; const b2 = data[2]; if (b2 >= 0x40 && b2 <= 0x47) { const lane = (b2 & 0x07) + 1; const ticks = ((data[8]<<24)¦(data[9]<<16)¦(data[10]<<8)¦data[11]) >>> 0; const secs = (ticks > 500 && ticks < 36000000) ? ticks / 1000 : 0; if (secs <= 0) return; this.emitTiming('T1', lane, secs); return; } if (b2 >= 0x20 && b2 <= 0x27) { // UNVERIFIED range — see contradiction note above const lane = (b2 & 0x07) + 1; const ticks = ((data[8]<<24)¦(data[9]<<16)¦(data[10]<<8)¦data[11]) >>> 0; const secs = (ticks > 500 && ticks < 36000000) ? ticks / 1000 : 0; if (secs <= 0) return; this.emitTiming('T2', lane, secs); return; } if (b2 >= 0xa0 && b2 <= 0xa7) return; // companion metadata, ignored }

# 10. Starter Gun Signal — ASCII Path (CMD 0x81)

VERIFIED structure, UNVERIFIED checksum byte:

01 81 0C 71 01 11 39 38 02 53 54 41 52 54 04 E8   -> "START"

01 81 0E 6F 01 11 39 38 02 53 54 41 52 54 5F 31 04 E8   -> "START_1"

Followed by TLH (Total Lap History) and SLH (Split Lap History) lines for each lane, and a heat-completion marker (SLH| 0|-1|-1|).

## Chronological Test Runs — as reported, with reliability flag

| Run # | Timestamp | Lanes Reported Active | Reliability |
| --- | --- | --- | --- |
| 1 | 16:22:38 | 3, 5, 8 | FLAGGED — identical to Runs 3, 4, 5 |
| 2 | 16:22:45 | 2, 4, 6, 8 | Distinct from others — no flag |
| 3 | 16:22:52 | 3, 5, 8 | FLAGGED — identical to Runs 1, 4, 5 |
| 4 | 16:22:59 | 3, 5, 8 | FLAGGED — identical to Runs 1, 3, 5 |
| 5 | 16:23:06 | 3, 5, 8 | FLAGGED — identical to Runs 1, 3, 4 |
| 6 | 16:23:13 | Heat finished (SLH -1/-1) | Distinct — no flag |

DATA QUALITY FLAG Runs 1, 3, 4, and 5 all report the identical lane set ("3, 5, 8") despite being described as separate test firings roughly 7 seconds apart. Genuinely independent tests producing identical results four times in a row is unlikely and suggests a copy-paste or extraction error in the original analysis rather than real per-run data. Treat this specific table as needing re-extraction from the raw capture before relying on it.

# 11. Starter Gun Signal — Binary Path (CMD 0x41)

PARTIALLY VERIFIED — only 2 real samples captured to date:

Fire 1:  01 41 11 ac 00 06 4c 60 00 00 00 00 ...

Fire 2:  01 41 11 ac 00 11 4c 60 00 00 00 00 ...

Constant across both: 01 41 11 ac ?? ?? 4c 60 — proposed as the detection signature. The changing byte (06 -> 11) is the same global sequence counter used across all frame types, not a gun-specific counter. Exact time-field position within this frame has not been confidently isolated — needs a dedicated multi-fire test (4-5 isolated gun fires, nothing else triggered) to confirm, the same methodology used successfully for lane decoding.

function isStarterGunEvent(f) { return f[0]===0x01 && f[1]===0x41 && f[2]===0x11 && f[3]===0xAC && f[6]===0x4C && f[7]===0x60; }

# 12. Sensor-Specific Contact Debouncing — as provided

private lastTouchTimestampByLaneAndMethod: { [key: string]: number } = {}; private emit(event: TimingEvent) { if (event.type === 'SPLIT' ¦¦ event.type === 'FINISH') { const lane = event.lane; const method = event.timingMethod ¦¦ 'T1'; if (lane && lane >= 1 && lane <= 8) { const now = Date.now(); const key = `${lane}_${method}`; const lastTouch = this.lastTouchTimestampByLaneAndMethod[key] ¦¦ 0; if (now - lastTouch < 400) return; this.lastTouchTimestampByLaneAndMethod[key] = now; } } this.callbacks.forEach(cb => cb(event)); }

RELATED BUT DISTINCT FROM THE VERIFIED DEDUP FINDING Joint captures independently confirmed a separate duplicate-frame phenomenon at ~50ms (multiple sub-type bytes for one physical tap). The 400ms value here was not derived from those captures — likely intentionally larger to also absorb genuine mechanical bounce, which is reasonable, just a different number from the ~50ms frame-duplication window measured directly.

# 13. Real-Time T1 vs T2 Arbitration — as provided

Depends entirely on the unverified/contradictory T2 hardware range from Section 9. The arbitration logic itself (software state machine) is independent of that and can be reviewed on its own merits.

## Arbitration Implementation — as provided

let isT2PendingWindowActive = false; if (timingMethod === 'T1') { if (pendingT2TimeoutsRef.current[laneNum]) { isT2PendingWindowActive = true; clearTimeout(pendingT2TimeoutsRef.current[laneNum].timeoutId); delete pendingT2TimeoutsRef.current[laneNum]; } lastTouchByLaneRef.current[laneNum] = { method: 'T1', time: event.time, timestamp: now }; } if (timingMethod === 'T2') { if (lastTouch && lastTouch.method === 'T1' && (now - lastTouch.timestamp < 2500)) return; if (pendingT2TimeoutsRef.current[laneNum]) clearTimeout(pendingT2TimeoutsRef.current[laneNum].timeoutId); const timeoutId = setTimeout(() => { delete pendingT2TimeoutsRef.current[laneNum]; }, 2000); pendingT2TimeoutsRef.current[laneNum] = { time: event.time, timestamp: now, timeoutId }; lastTouchByLaneRef.current[laneNum] = { method: 'T2', time: event.time, timestamp: now }; }

# 14. Multi-Heat & Reset State Management — as provided

- On Race Start: markRaceStarted() locks hardware clock reference; clears pending T2 timers and lane touch records; clears timingMethod on all lanes.

- On Heat/Event Change: clears pending T2 timers and lane touch records.

- On Timer Reset: resetRaceStartHardwareTime() resets hardware offsets; clears all pending timers and lane touch records; resets timingMethod on all 8 lanes.

# 15. Summary Matrix of Arbitration Outcomes — as provided

| Sequence | Touch Interval | Time Recorded | Badge |
| --- | --- | --- | --- |
| T1 first, T2 second | Any time | T1 Time | T1 |
| T2 first, T1 second | Within 2.0s | T1 Time (Overridden) | T1 |
| T2 first, T1 second | After 2.0s | T2 Time (Preserved) | T2 |
| T2 first, no T1 | 2.0+ seconds | T2 Time | T2 |

# 16. Hardware Fixes & Protocol Solutions — Implementation Notes

## Issue 1: Green light turning off after refresh

Claimed root cause: Clock Sync steps (12-13) reset the internal clock counter, disarming the device. Fix applied: explicit trailing armLanes(true) call after clock sync in sendAresInit(). This is a plausible, testable software fix; the underlying causal claim about clock sync disarming the device has not been independently verified the way the Section 5 LED-trigger claim was tested and disproven — worth the same controlled test if it becomes important.

## Issue 2: Phantom gun fire during initialization

Claimed root cause: startup status packets contain 0x4C flags misidentified as gun fire. Fix applied: 2.5s suppression window after handshake. Reasonable defensive measure regardless of root-cause certainty.

## Issue 3: Touchpad time drift spikes

Claimed root cause: absolute console uptime recorded instead of race-relative time when start offset is 0. Fix applied: drift protection comparing touch time against expected race-elapsed time, correcting if the difference exceeds 10 seconds.

if (timerStatusRef.current === 'RUNNING' && timerStartRef.current > 0) { const currentRaceElapsed = (now - timerStartRef.current) / 1000; if (Math.abs(touchTime - currentRaceElapsed) > 10.0) { touchTime = Number(currentRaceElapsed.toFixed(2)); } }

# 17. Net Elapsed Time Math

Net Elapsed Time = T_touchpad − T_race_start. Store T_race_start when the START signal is received; subtract from each raw touchpad timestamp to get race-relative time. Concept is sound and consistent with verified timestamp decoding in Section 8.

# 18. Raw Serial Feed Color-Coding Specification (UI, no hardware verification needed)

| Log Type | Trigger String | Color | Weight |
| --- | --- | --- | --- |
| Race Start/Stop | "Started race clock." / "Stopped race clock." | #ef4444 (Red) | 900 |
| System Sync/Ready | "100% READY" / "Synchronized with ARES" | #fbbf24 (Amber) | 800 |
| Touchpad Event | "[TOUCHPAD" / "TOUCH (" | #38bdf8 (Cyan) | 600 |
| Default Log | Status/keepalive packets | #a7f3d0 (Emerald) | 400 |

# 19. Standard Operating Procedure — as provided

- Terminal 1: npm run bridge — starts the Node serial bridge on ws://localhost:8787.

- Terminal 2: npm run dev — starts the web app.

- Open http://localhost:5173 in Chrome.

- Verify: amber "Synchronized" log line, green COM PORT CONNECTED pill, physical ready light on.

- Manual controls: "Arm Start Light" sends CMD 0x16; "Disarm Start Light" sends the disarm command.

# 20. Master Verification Checklist

- [ VERIFIED ] Physical serial parameters (9600 8N1, RTS/DTR cleared).

- [ VERIFIED ] 14-step connection handshake byte sequence.

- [ CORRECTED ] No single command arms/triggers the blink — continuous valid traffic keeps device active, not one discrete step.

- [ VERIFIED ] T1 lane formula, byte range 0x40-0x47, cross-matched against on-screen times including reversed finish order.

- [ VERIFIED ] Millisecond time decoding, 4-byte big-endian ÷ 1000.

- [ VERIFIED ] Frame-level duplicate suppression needed (~50ms window observed).

- [ PARTIALLY VERIFIED ] CMD 0x41 as starter signature — only 2 samples, time field position not confirmed.

- [ CONTRADICTION — UNRESOLVED ] T2 byte range: 0x20-0x27 vs 0x81-0x88/0xC1-0xC8 — needs an isolated T2-only capture.

- [ UNVERIFIED ] Checksum formula — needs testing against real frame bytes.

- [ UNVERIFIED ] Secondary lane range 0x80-0xC7 — never observed.

- [ UNVERIFIED ] 0x32 clock-drop gun-trigger detection; 0x25/0x26 disarm commands.

- [ DATA QUALITY FLAG ] Starter test run log — 4 of 6 runs show identical lane results, likely needs re-extraction.

- [ IMPLEMENTATION, NOT HARDWARE-VERIFIED ] WebSocket bridge architecture, refresh/phantom-fire/drift fixes — internally consistent, worth confirming root causes with the same controlled-test methodology used elsewhere in this document.